eszközök és konfigurációk

DOLG-PC:

DOLG-PR:

KNYV-PC:

KNYV-SVR:

KNYV-PR:

DOLG-SW:

DOLG-SW#show running-config

Building configuration...

Current configuration : 2571 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname DOLG-SW

!

enable secret 5 $1$mERr$4sQaZg8gZWvXUt4hXKENY1

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface Port-channel2

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/2

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/3

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 1 mode passive

!

interface FastEthernet0/4

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 1 mode passive

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/8

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/9

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/10

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 082543620E491F471B205C0A2078287D29

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08051C420E2F110E2658000A793F147C666015

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08051C420E2F110E2658000A793F147C666015

logging synchronous

login

transport input ssh

!

!

!

!

end

KNYV-SW:

KNYV-SW#show run

Building configuration...

Current configuration : 2551 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname KNYV-SW

!

enable secret 5 $1$mERr$L/SRVeUhrXMktV.USsfWe0

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

interface Port-channel2

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface Port-channel3

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 2 mode passive

!

interface FastEthernet0/2

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 2 mode passive

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/6

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/7

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/8

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/9

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/10

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 080A1C40100F561B17182A570603701B09

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 080A1C40300F561B260E002A791F

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 080A1C40300F561B260E002A791F

logging synchronous

login

transport input ssh

!

!

!

!

end

BP-SW:

BP-SW#show run

Building configuration...

Current configuration : 2299 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname BP-SW

!

enable secret 5 $1$mERr$lSWvxesqZ5vL2jCry9lyA1

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 10,20,30,100 priority 4096

!

interface Port-channel1

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface Port-channel3

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/4

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/5

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 3 mode passive

!

interface FastEthernet0/6

switchport trunk allowed vlan 10,20,30,100

switchport mode trunk

channel-group 3 mode passive

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 08037C7D1E4811140134075404317404

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08035C4B5C0D33032B3438572625

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08035C4B5C0D33032B3438572625

logging synchronous

login

transport input ssh

!

!

!

!

end

BP-RT:

BP-RT#show run

Building configuration...

Current configuration : 2362 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 8

!

hostname BP-RT

!

!

!

enable secret 5 $1$mERr$r4GaONU0/xJ.8eITME7tY.

!

!

!

clock timezone CET 1

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX10179WL1-

!

!

!

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 5

lifetime 1800

!

crypto isakmp key HORIZONKULCS address 203.0.113.10

!

!

!

crypto ipsec transform-set BP-DEB esp-aes esp-sha-hmac

!

crypto map HORIZONMAP 10 ipsec-isakmp

set peer 203.0.113.10

set pfs group5

set security-association lifetime seconds 900

set transform-set BP-DEB

match address VPNLIST

!

!

!

!

!

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

ip address 200.200.200.1 255.255.255.248

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

!

interface FastEthernet0/1.10

encapsulation dot1Q 10

ip address 192.168.1.1 255.255.255.224

ip helper-address 192.168.1.3

standby 10 ip 192.168.1.30

standby 10 priority 110

standby 10 preempt

!

interface FastEthernet0/1.20

encapsulation dot1Q 20

ip address 192.168.1.33 255.255.255.224

ip helper-address 192.168.1.3

standby 20 ip 192.168.1.62

standby 20 priority 110

standby 20 preempt

!

interface Serial0/0/0

ip address 203.0.113.2 255.255.255.252

crypto map HORIZONMAP

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 1.1.1.1

log-adjacency-changes

network 200.200.200.0 0.0.0.7 area 0

network 192.168.1.0 0.0.0.31 area 0

network 192.168.1.32 0.0.0.31 area 0

network 203.0.113.0 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended VPNLIST

permit ip 203.0.113.0 0.0.0.3 203.0.113.8 0.0.0.3

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 08037C7C1D325519085B00517D7876

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 08037C713B0D3A21061233307927

logging synchronous

login

transport input ssh

!

!

!

end

BP-RT2:

BP-RT2#show run

Building configuration...

Current configuration : 1680 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 8

!

hostname BP-RT2

!

!

!

enable secret 5 $1$mERr$5tYBF.2T1TOuv..KvRLrk1

!

!

!

clock timezone CET 1

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX10172V33-

!

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

no ip address

duplex auto

speed auto

!

interface FastEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.1.2 255.255.255.224

standby 10 ip 192.168.1.30

!

interface FastEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.1.34 255.255.255.224

standby 20 ip 192.168.1.62

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 203.0.113.6 255.255.255.252

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 2.2.2.2

log-adjacency-changes

network 192.168.1.32 0.0.0.31 area 0

network 192.168.1.0 0.0.0.31 area 0

network 203.0.113.4 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 08034D7E1D26370317395E3B01240A223639

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0813581C3F0D1C28260E202A793F

logging synchronous

login

transport input ssh

!

!

!

end

IGT-TF:

IGT-TF#show run

ASA Version 9.6(1)

!

hostname IGT-TF

enable password Vy0wEO5/w193Aqbp encrypted

names

!

interface GigabitEthernet1/1

nameif outside

security-level 0

ip address 200.200.200.2 255.255.255.248

!

interface GigabitEthernet1/2

nameif inside

security-level 100

ip address 192.168.1.65 255.255.255.240

!

interface GigabitEthernet1/3

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/4

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/5

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/6

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/7

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/8

no nameif

no security-level

no ip address

shutdown

!

interface Management1/1

management-only

no nameif

no security-level

no ip address

shutdown

!

!

!

access-list PERMIT extended permit ip 192.168.2.0 255.255.255.224 192.168.1.64 255.255.255.240

access-list PERMIT extended permit ip 192.168.1.32 255.255.255.224 192.168.1.64 255.255.255.240

!

!

access-group PERMIT out interface inside

aaa authentication ssh console LOCAL

!

username admin password 3bl4Ll3beGamEvh4 encrypted

!

class-map inspection_default

match default-inspection-traffic

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum 512

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect tftp

!

service-policy global_policy global

!

telnet timeout 5

ssh 192.168.1.64 255.255.255.240 inside

ssh timeout 3

!

!

crypto ipsec ikev1 transform-set BP-DEB esp-aes esp-sha-hmac

!

crypto map HORIZONMAP 10 set ikev1 transform-set BP-DEB

!

router ospf 1

router-id 5.5.5.5

log-adjacency-changes

network 192.168.1.64 255.255.255.240 area 0

network 200.200.200.0 255.255.255.248 area 0

!

IGT-SW:

IGT-SW(config)#do sh run

Building configuration...

Current configuration : 1530 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IGT-SW

!

enable secret 5 $1$mERr$a7Tr9f4W6HIrZRa6/5NpF/

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 08086B7A36325519085B003B227F1712

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08086B7A1F0D1C3B43055F

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08086B7A1F0D1C3B43055F

logging synchronous

login

transport input ssh

!

!

!

!

end

IGT-SVR:

IGT-PC:

ISP:

ISP#show run

Building configuration...

Current configuration : 1359 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname ISP

!

!

!

!

!

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX10174AHE-

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

no ip address

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 203.0.113.1 255.255.255.252

clock rate 2000000

!

interface Serial0/0/1

ip address 203.0.113.5 255.255.255.252

clock rate 2000000

!

interface Serial0/1/0

ip address 203.0.113.9 255.255.255.252

clock rate 2000000

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/2/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/2/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/3/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 3.3.3.3

log-adjacency-changes

network 203.0.113.0 0.0.0.3 area 0

network 203.0.113.4 0.0.0.3 area 0

network 203.0.113.8 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end

DEB-RT:

DEB-RT(config)#do show run

Building configuration...

Current configuration : 2043 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 8

!

hostname DEB-RT

!

!

!

enable secret 5 $1$mERr$vMcrBN/DfPzNMx8ZDobeB0

!

!

!

clock timezone CET 1

!

!

!

!

ip cef

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX1017J63X-

!

!

!

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 5

lifetime 1800

!

crypto isakmp key HORIZONKULCS address 203.0.113.2

!

!

!

crypto ipsec transform-set BP-DEB esp-aes esp-sha-hmac

!

crypto map HORIZONMAP 10 ipsec-isakmp

set peer 203.0.113.2

set pfs group5

set security-association lifetime seconds 900

set transform-set BP-DEB

match address VPNLIST

!

!

!

!

!

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

ip address 192.168.2.1 255.255.255.192

duplex auto

speed auto

ipv6 address FE80::4 link-local

ipv6 address 2001:DB8:ACAD:40::1/64

!

interface FastEthernet0/1

ip address 110.110.110.1 255.255.255.248

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

ip address 203.0.113.10 255.255.255.252

crypto map HORIZONMAP

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 4.4.4.4

log-adjacency-changes

network 192.168.2.0 0.0.0.63 area 0

network 203.0.113.8 0.0.0.3 area 0

network 110.110.110.0 0.0.0.7 area 0

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended VPNLIST

permit ip 203.0.113.8 0.0.0.3 203.0.113.0 0.0.0.3

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 08051F4C362B1128395B021E7A27

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 08051F4C362B1128241F153B1E7828

logging synchronous

login

transport input ssh

!

!

!

end

DEB-SW:

DEB-SW(config)#do show run

Building configuration...

Current configuration : 1568 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname DEB-SW

!

enable secret 5 $1$mERr$KyPC0T7sVvvhr8Us5OAP..

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 08051F4C362A12031103332F7A251E273F

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08051F4C3F0D1C23410702573E3B707D66223017

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08051F4C3F0D1C23410702573E3B707D66223017

logging synchronous

login

transport input ssh

!

!

!

!

end

DEB-PR:

DEB-Admin:

DEB-PC:

DEB-SVR:

DEB-WR:

DEB-LTP:

DEB-R-UGYFEL:

VOICE>ena

VOICE#show run

Building configuration...

Current configuration : 1788 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname VOICE

!

!

!

!

ip dhcp excluded-address 196.60.60.65

ip dhcp excluded-address 196.60.60.193

!

ip dhcp pool voice

network 196.60.60.64 255.255.255.192

default-router 196.60.60.65

option 150 ip 196.60.60.65

dns-server 192.168.1.66

domain-name horizonbank.hu

ip dhcp pool client

network 196.60.60.192 255.255.255.192

default-router 196.60.60.193

dns-server 192.168.1.66

domain-name horizonbank.hu

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX10174DAK-

!

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

no ip address

duplex auto

speed auto

!

interface FastEthernet0/0.30

encapsulation dot1Q 30

ip address 196.60.60.65 255.255.255.192

!

interface FastEthernet0/0.40

encapsulation dot1Q 40

ip address 196.60.60.193 255.255.255.192

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/3/0

no ip address

clock rate 2000000

!

interface Serial0/3/1

no ip address

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

telephony-service

max-ephones 3

max-dn 3

ip source-address 196.60.60.65 port 2000

auto assign 1 to 9

!

ephone-dn 1

number 1111

!

ephone-dn 2

number 2222

!

ephone-dn 3

number 3333

!

ephone 1

device-security-mode none

mac-address 00D0.BAD7.8036

type 7960

button 1:1

!

ephone 2

device-security-mode none

mac-address 000A.415A.D4B7

type 7960

button 1:2

!

ephone 3

device-security-mode none

mac-address 0000.0C29.D08D

type 7960

button 1:3

!

line con 0

logging synchronous

!

line aux 0

!

line vty 0 4

login

!

!

!

end

DEB-SW-UGYFEL:

SWVOICE#show run

Building configuration...

Current configuration : 1415 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname SWVOICE

!

!

!

no ip domain-lookup

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport trunk allowed vlan 1,30,40

switchport mode trunk

!

interface FastEthernet0/2

switchport access vlan 40

switchport mode access

switchport voice vlan 30

!

interface FastEthernet0/3

switchport access vlan 40

switchport mode access

switchport voice vlan 30

!

interface FastEthernet0/4

switchport access vlan 40

switchport mode access

switchport voice vlan 30

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

logging synchronous

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

end