Budai János, Budai Bálint eszközök konfigurációi

Tartalom

Routerek	13

BP-RT1:	13

BP-RT2:	17

DEB-RT:	19

VOICE-RT:	22

Switchek	25

BP-SW:	25

KNYV-SW:	28

DOLG-SW:	31

IGT-SW:	34

DEB-SW:	37

Switch0:	39

Szerverek	41

KNYV-SVR: 	42

IGT-SVR:	44

DEB-SVR: 	45

PC-k:	47

DOLG-PC:	47

KNYV-PC:	48

IGT-Admin: 	49

DEB-Admin: 	50

DEB-PC:	51

PC0: 	52

PC1: 	53

PC2: 	54

Routerek

BP-RT1:

Building configuration...

Current configuration : 2869 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 8

!

hostname BP-RT

!

!

!

enable secret 5 $1$mERr$r4GaONU0/xJ.8eITME7tY.

!

!

!

clock timezone CET 1

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX10179WL1-

!

!

!

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 5

lifetime 1800

!

crypto isakmp key HORIZONKULCS address 203.0.113.34

!

!

!

crypto ipsec transform-set BP-DEB esp-aes esp-sha-hmac

!

crypto map HORIZONMAP 10 ipsec-isakmp

set peer 203.0.113.34

set pfs group5

set security-association lifetime seconds 900

set transform-set BP-DEB

match address VPNLIST

!

!

!

!

!

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

ip address 200.200.200.1 255.255.255.248

ip nat inside

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

!

interface FastEthernet0/1.10

encapsulation dot1Q 10

ip address 192.168.1.1 255.255.255.224

ip helper-address 192.168.1.3

ip nat inside

standby 10 ip 192.168.1.30

standby 10 priority 110

standby 10 preempt

!

interface FastEthernet0/1.20

encapsulation dot1Q 20

ip address 192.168.1.33 255.255.255.224

ip helper-address 192.168.1.3

ip nat inside

standby 20 ip 192.168.1.62

standby 20 priority 110

standby 20 preempt

!

interface Serial0/0/0

ip address 203.0.113.2 255.255.255.240

ip nat outside

crypto map HORIZONMAP

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 1.1.1.1

log-adjacency-changes

network 200.200.200.0 0.0.0.7 area 0

network 192.168.1.0 0.0.0.31 area 0

network 192.168.1.32 0.0.0.31 area 0

network 203.0.113.0 0.0.0.15 area 0

!

ip nat pool KONYVELESNAT 203.0.113.4 203.0.113.14 netmask 255.255.255.240

ip nat inside source list NAT_LIST pool KONYVELESNAT

ip nat inside source static 192.168.1.3 203.0.113.3

ip classless

!

ip flow-export version 9

!

!

ip access-list extended VPNLIST

permit ip 192.168.1.0 0.0.0.31 192.168.2.0 0.0.0.63

permit ip 192.168.1.32 0.0.0.31 192.168.2.0 0.0.0.63

ip access-list extended NAT_LIST

deny ip 192.168.1.0 0.0.0.31 192.168.2.0 0.0.0.63

deny ip 192.168.1.32 0.0.0.31 192.168.2.0 0.0.0.63

permit ip 192.168.1.0 0.0.0.31 any

permit ip 192.168.1.32 0.0.0.31 any

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 08037C7C1D325519085B00517D7876

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 08037C713B0D3A21061233307927

logging synchronous

login

transport input ssh

!

!

!

end

BP-RT2:

Building configuration...

Current configuration : 1905 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 8

!

hostname BP-RT2

!

!

!

enable secret 5 $1$mERr$5tYBF.2T1TOuv..KvRLrk1

!

!

!

clock timezone CET 1

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX10172V33-

!

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

no ip address

ip nat inside

duplex auto

speed auto

!

interface FastEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.1.2 255.255.255.224

standby 10 ip 192.168.1.30

!

interface FastEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.1.34 255.255.255.224

standby 20 ip 192.168.1.62

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 203.0.113.18 255.255.255.240

ip nat outside

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 2.2.2.2

log-adjacency-changes

network 192.168.1.32 0.0.0.31 area 0

network 192.168.1.0 0.0.0.31 area 0

network 203.0.113.16 0.0.0.15 area 0

!

ip nat pool DOLGOZOINAT 203.0.113.17 203.0.113.30 netmask 255.255.255.240

ip nat inside source list 101 pool DOLGOZOINAT

ip classless

!

ip flow-export version 9

!

!

access-list 101 permit ip 203.0.113.16 0.0.0.15 203.0.113.32 0.0.0.15

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 08034D7E1D26370317395E3B01240A223639

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 0813581C3F0D1C28260E202A793F

logging synchronous

login

transport input ssh

!

!

!

end

DEB-RT:

Building configuration...

Current configuration : 2121 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 8

!

hostname DEB-RT

!

!

!

enable secret 5 $1$mERr$vMcrBN/DfPzNMx8ZDobeB0

!

!

!

clock timezone CET 1

!

!

!

!

ip cef

ipv6 unicast-routing

!

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX1017J63X-

!

!

!

crypto isakmp policy 10

encr aes 256

authentication pre-share

group 5

lifetime 1800

!

crypto isakmp key HORIZONKULCS address 203.0.113.2

!

!

!

crypto ipsec transform-set BP-DEB esp-aes esp-sha-hmac

!

crypto map HORIZONMAP 10 ipsec-isakmp

set peer 203.0.113.2

set pfs group5

set security-association lifetime seconds 900

set transform-set BP-DEB

match address VPNLIST

!

!

!

!

!

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

ip address 192.168.2.1 255.255.255.192

duplex auto

speed auto

ipv6 address 2001:DB8:ACAD:40::1/64

!

interface FastEthernet0/1

ip address 192.168.2.65 255.255.255.252

duplex auto

speed auto

!

interface Serial0/0/0

ip address 120.120.120.1 255.255.255.252

clock rate 2000000

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Serial0/1/0

ip address 203.0.113.34 255.255.255.240

crypto map HORIZONMAP

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 4.4.4.4

log-adjacency-changes

network 192.168.2.0 0.0.0.63 area 0

network 203.0.113.32 0.0.0.15 area 0

network 120.120.120.0 0.0.0.3 area 0

network 192.168.2.64 0.0.0.3 area 0

!

ip classless

!

ip flow-export version 9

!

!

ip access-list extended VPNLIST

permit ip 192.168.2.0 0.0.0.63 192.168.1.0 0.0.0.31

permit ip 192.168.2.0 0.0.0.63 192.168.1.32 0.0.0.31

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

!

line con 0

exec-timeout 5 0

password 7 08051F4C362B1128395B021E7A27

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 08051F4C362B1128241F153B1E7828

logging synchronous

login

transport input ssh

!

!

!

end

VOICE-RT:

Current configuration : 2331 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

security passwords min-length 8

!

hostname VOICE-RT

!

!

!

enable secret 5 $1$mERr$U0W1pV0rfAy5mjOBoUH42.

!

!

ip dhcp excluded-address 196.60.60.65

ip dhcp excluded-address 196.60.60.193

!

ip dhcp pool voice

network 196.60.60.64 255.255.255.192

default-router 196.60.60.65

option 150 ip 196.60.60.65

dns-server 192.168.1.66

domain-name horizonbank.hu

ip dhcp pool client

network 196.60.60.192 255.255.255.192

default-router 196.60.60.193

dns-server 192.168.1.66

domain-name horizonbank.hu

clock timezone CET 1

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2811/K9 sn FTX10174DAK-

!

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

no ip address

duplex auto

speed auto

!

interface FastEthernet0/0.50

encapsulation dot1Q 50

ip address 196.60.60.65 255.255.255.192

!

interface FastEthernet0/0.60

encapsulation dot1Q 60

ip address 196.60.60.193 255.255.255.192

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/3/0

ip address 120.120.120.2 255.255.255.252

!

interface Serial0/3/1

no ip address

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

router-id 6.6.6.6

log-adjacency-changes

network 120.120.120.0 0.0.0.3 area 0

network 196.60.60.64 0.0.0.63 area 0

network 196.60.60.192 0.0.0.63 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

!

telephony-service

max-ephones 3

max-dn 3

ip source-address 196.60.60.65 port 2000

auto assign 1 to 9

!

ephone-dn 1

number 1111

!

ephone-dn 2

number 2222

!

ephone-dn 3

number 3333

!

ephone 1

device-security-mode none

mac-address 00D0.BAD7.8036

type 7960

button 1:1

!

ephone 2

device-security-mode none

mac-address 000A.415A.D4B7

type 7960

button 1:2

!

ephone 3

device-security-mode none

mac-address 0000.0C29.D08D

type 7960

button 1:3

!

line con 0

exec-timeout 5 0

password 7 08021C401F495414171B0D17397A767B70

logging synchronous

login

!

line aux 0

!

line vty 0 4

exec-timeout 5 0

password 7 081758571F495414171B0D17397A767B70

logging synchronous

login

transport input ssh

!

!

!

end

Switchek

BP-SW:

Building configuration...

Current configuration : 2544 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname BP-SW

!

enable secret 5 $1$mERr$lSWvxesqZ5vL2jCry9lyA1

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 10,20,100 priority 4096

!

interface Port-channel1

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

!

interface Port-channel3

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/4

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/5

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 3 mode passive

!

interface FastEthernet0/6

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 3 mode passive

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 08037C7D1E4811140134075404317404

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08035C4B5C0D33032B3438572625

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08035C4B5C0D33032B3438572625

logging synchronous

login

transport input ssh

!

!

!

!

end

KNYV-SW:

Building configuration...

Current configuration : 2835 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname KNYV-SW

!

enable secret 5 $1$mERr$L/SRVeUhrXMktV.USsfWe0

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

interface Port-channel1

!

interface Port-channel2

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

switchport nonegotiate

!

interface Port-channel3

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

switchport nonegotiate

channel-group 2 mode passive

!

interface FastEthernet0/2

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

switchport nonegotiate

channel-group 2 mode passive

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/6

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/7

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/8

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/9

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/10

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 080A1C40100F561B17182A570603701B09

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 080A1C40300F561B260E002A791F

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 080A1C40300F561B260E002A791F

logging synchronous

login

transport input ssh

!

!

!

!

end

DOLG-SW:

Building configuration...

Current configuration : 2829 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname DOLG-SW

!

enable secret 5 $1$mERr$4sQaZg8gZWvXUt4hXKENY1

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

!

interface Port-channel2

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

switchport nonegotiate

!

interface FastEthernet0/1

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

switchport nonegotiate

channel-group 2 mode active

!

interface FastEthernet0/2

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

switchport nonegotiate

channel-group 2 mode active

!

interface FastEthernet0/3

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 1 mode passive

!

interface FastEthernet0/4

switchport trunk native vlan 100

switchport trunk allowed vlan 10,20,100

switchport mode trunk

channel-group 1 mode passive

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/8

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/9

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/10

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 082543620E491F471B205C0A2078287D29

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08051C420E2F110E2658000A793F147C666015

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08051C420E2F110E2658000A793F147C666015

logging synchronous

login

transport input ssh

!

!

!

!

end

IGT-SW:

Building configuration...

Current configuration : 1530 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname IGT-SW

!

enable secret 5 $1$mERr$a7Tr9f4W6HIrZRa6/5NpF/

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 08086B7A36325519085B003B227F1712

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08086B7A1F0D1C3B43055F

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08086B7A1F0D1C3B43055F

logging synchronous

login

transport input ssh

!

!

!

!

end

DEB-SW:

Building configuration...

Current configuration : 1568 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname DEB-SW

!

enable secret 5 $1$mERr$KyPC0T7sVvvhr8Us5OAP..

!

!

!

clock timezone CET 1

!

no ip domain-lookup

ip domain-name horizonbank.hu

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

!

interface FastEthernet0/2

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

banner motd ^C Az illetektelen hozzaferes TILOS! ^C

!

!

!

line con 0

password 7 08051F4C362A12031103332F7A251E273F

logging synchronous

login

exec-timeout 5 0

!

line vty 0 4

exec-timeout 5 0

password 7 08051F4C3F0D1C23410702573E3B707D66223017

logging synchronous

login

transport input ssh

line vty 5 15

exec-timeout 5 0

password 7 08051F4C3F0D1C23410702573E3B707D66223017

logging synchronous

login

transport input ssh

!

!

!

!

end

Switch0:

Building configuration...

Current configuration : 1415 bytes

!

version 15.0

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname SWVOICE

!

!

!

no ip domain-lookup

!

!

!

spanning-tree mode pvst

spanning-tree extend system-id

!

interface FastEthernet0/1

switchport trunk allowed vlan 1,50,60

switchport mode trunk

!

interface FastEthernet0/2

switchport access vlan 60

switchport mode access

switchport voice vlan 50

!

interface FastEthernet0/3

switchport access vlan 60

switchport mode access

switchport voice vlan 50

!

interface FastEthernet0/4

switchport access vlan 60

switchport mode access

switchport voice vlan 50

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

logging synchronous

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

end

Szerverek

KNYV-SVR:

IGT-SVR:

DEB-SVR:

PC-k:

DOLG-PC:

KNYV-PC:

IGT-Admin:

DEB-Admin:

DEB-PC:

PC0:

PC1:

PC2:

IP telefonok:

IP Phone0:

IP Phone1:

IP Phone2:

Nyomtatók:

KNYV-PR:

DOLG-PR:

DEB-PR:

Wireless router:

DEB-WR:

Tűzfal:

ASA0:

ASA Version 9.6(1)

!

hostname IGT-TF

enable password 9q7yxKHQh6upHHvv encrypted

names

!

interface GigabitEthernet1/1

nameif outside

security-level 0

ip address 200.200.200.2 255.255.255.248

!

interface GigabitEthernet1/2

nameif inside

security-level 100

ip address 192.168.1.65 255.255.255.240

!

interface GigabitEthernet1/3

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/4

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/5

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/6

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/7

no nameif

no security-level

no ip address

shutdown

!

interface GigabitEthernet1/8

no nameif

no security-level

no ip address

shutdown

!

interface Management1/1

management-only

no nameif

no security-level

no ip address

shutdown

!

object network IGT-ADMIN

host 192.168.1.67

nat (inside,outside) static 200.200.200.4

object network IGT-SVR

host 192.168.1.66

nat (inside,outside) static 200.200.200.3

!

route outside 0.0.0.0 0.0.0.0 200.200.200.1 1

!

access-list PERMIT extended permit ip 192.168.2.0 255.255.255.192 192.168.1.64 255.255.255.240

access-list PERMIT extended permit ip 192.168.1.0 255.255.255.224 192.168.1.64 255.255.255.240

access-list PERMIT extended permit ip 192.168.1.32 255.255.255.224 192.168.1.64 255.255.255.240

access-list ICMPALL extended permit icmp any any

access-list outside-network extended permit ip any object IGT-ADMIN

access-list outside-network extended permit ip any object IGT-SVR

!

!

access-group PERMIT out interface inside

access-group outside-network in interface outside

access-group ICMPALL in interface inside

aaa authentication ssh console LOCAL

!

username admin password 3bl4Ll3beGamEvh4 encrypted

!

class-map inspection_default

match default-inspection-traffic

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum 512

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect tftp

!

service-policy global_policy global

!

telnet timeout 5

ssh 192.168.1.64 255.255.255.240 inside

ssh timeout 3

!

!

!

!

router ospf 1

router-id 5.5.5.5

log-adjacency-changes

network 192.168.1.64 255.255.255.240 area 0

network 200.200.200.0 255.255.255.248 area 0

!